User Manual

# Smart Trolley Using ESP32, RFID, Firebase & BLE Beacons

## 🔹 1. Introduction

The Smart Trolley system automates shopping by integrating RFID item identification, Firebase cloud database, BLE beacon navigation, and QR-based payment. It eliminates manual billing and guides customers through aisles using Bluetooth signals.

## 🔹 2. System Overview

| ### Component | ### Function |
| --- | --- |
| #### ESP32 | Central controller managing sensors, display, and cloud communication |
| #### RFID Module (MFRC522) | Reads item tags and identifies products |
| #### IR Sensor | Detects item addition or removal |
| #### OLED Display | Shows item details, total bill, navigation, and QR code |
| #### BLE Beacon | Transmits signals for aisle navigation |
| #### Firebase Database | Stores item details and billing information |
| #### Push Buttons | Start/stop navigation and complete shopping |

## 🔹 3. Working Principle

- Each product has an RFID tag linked to its details in Firebase.

- When scanned, ESP32 retrieves item data and updates the bill.

- IR sensor confirms whether the item is added or removed.

- OLED displays item name, price, and total bill.

- BLE beacon navigation helps locate aisles using RSSI strength.

- Pressing the “Payment” button generates a QR code.

- After payment, Firebase updates the shopping summary on the webpage.

## 🔹 4. Hardware Setup

- Connect RFID module via SPI pins.

- Connect IR sensor, buttons, buzzer, and LEDs to GPIO pins.

- Connect OLED display via I2C (SDA, SCL).

- Power the trolley using a 5V battery or adapter.

- Place BLE beacons at aisle entrances.

## 🔹 5. Software Setup

- Install Arduino IDE and required libraries:

  - WiFi.h, FirebaseESP32.h, Adafruit_SSD1306.h, MFRC522.h, BLEDevice.h

- Configure Firebase with your project credentials.

- Upload the ESP32 code.

- Verify OLED display and RFID scanning.

- Test BLE beacon signal strength for navigation.

## 🔹 6. Operating Instructions

- Power on the trolley.

- Scan items using RFID reader.

- Observe OLED updates for item addition/removal.

- Press Navigation button to locate aisles.

- Press Payment button to generate QR code.

- Complete payment via UPI app.

- Confirm shopping summary on the webpage.

## 🔹 7. Troubleshooting

| ### Issue | ### Possible Cause | ### Solution |
| --- | --- | --- |
| #### OLED not displaying | Loose I2C connection | Check SDA/SCL wiring |
| #### RFID not scanning | Tag too far or damaged | Bring tag closer or replace |
| #### BLE navigation inaccurate | Weak beacon signal | Adjust beacon placement |
| #### Firebase not updating | Wi-Fi disconnected | Reconnect and retry |

## 🔹 8. Safety Precautions

- Avoid exposing sensors to moisture.

- Handle electronic components carefully.

- Ensure stable power supply to prevent resets.

## 🔹 9. Conclusion

The Smart Trolley system enhances shopping efficiency through IoT automation, real-time billing, and indoor navigation. It demonstrates the integration of hardware and cloud technologies for a seamless retail experience.